Absolute Motor Zero Positions:

Joint 1 and 4 Absolute Motor Zero Positions:

Joint 1 is aligned such that the end effector meets the edge of the table (CW Positive Looking Into Rotor)

Joint 4 is aligned to the lower physical limit.(CW Positive Looking Into Rotor)

Joints 2 and 3 Motor Zero Positions:

Joint 2 is aligned with the lower physical limit (CW Positive Looking Into Rotor)

Joint 3 is aligned with the textured face facing up (CW Positive Looking Into Rotor)

Joint 5 Motor Zero Position:

Conduit exit is rotated 90 CW from facing up. I.e. When looking at the robot from the front of the table, the conduit exit is pointing to the right.

Joint 6 and 7 Motor Zero Positions:

TBD; May or may not be needed? Could probably just set a virtual zero that makes the end effector zero straight out the end

Virtual Home, Zero Positions, and Joint Limits:

The Robot’s virtual home is shown above. This represents a shutdown safe position that is not susceptible to collapsing.

| Joint | Angle Limit Low (deg - multi) | Angle Limit High (deg - multi) | Virtual Home Angle (deg - multi) | Positive Direction Looking Into Rotor |
| --- | --- | --- | --- | --- |
| 1 | 0 | 210 | 106.3 | CW |
| 2 | 5 | 215 | 63.6 | CW |
| 3 | -30 | 225 | 86.8 | CW |
| 4 | 2.5 | 245 | 1 | CW |
| 5 | -30 | 210 | 89.23 | CCW |
| 6 |  |  |  | CCW |
| 7 |  |  |  | CCW |

Motor Specifications and Accelerations:

| Joints | Max Torque (Nm) | Rated Torque (Nm) | Gear Ratio | Max Speed (rpm) | Conservative Range (deg/s²) | Typical Range (deg/s²) | Aggressive Range (deg/s²) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| J1–J2 | 28 | 14 | 9 | 177 | 200–400 | 400–800 | 800–1200 |
| J3–J4 | 20 | 10 | 9 | 178 | 200–400 | 400–800 | 800–1200 |
| J5 | 4.5 | 2.5 | 10 | 260 | 300–600 | 600–1000 | 1000–1500 |
| J6–J7 | 2.5 | 1 | 10 | 253 | 300–600 | 600–1200 | 1200–1500 |